TRAXIS MANUFACTURING

Machine Utilization Report

Mon, Feb 2, 2026 — Wed, Feb 4, 2026

# Executive Summary

During the reporting period (3 working days), 4 of 5 monitored CNC machines were active. The shop achieved an average utilization of 15% across active machines (based on actual cutting time — spindle warmup and air spinning are excluded). Machines logged 17.3 cutting hours and 61.4 spindle-only hours out of 116 available hours during shift time (6:00 - 19:00).

| Shop Utilization: | 15% | Active Machines: | 4 of 5 |
| --- | --- | --- | --- |
| Hours Cutting: | 17.3h | Hours Running: | 78.7h |
| Hours Available: | 116h | Active Machines: | 4 of 5 |
| Green Target: | ≥ 70% | Warning Below: | < 50% |

# Utilization by Machine

## Detailed Breakdown

| ID | Machine | Cutting | Spindle | Total | Running | Available | Status |
| --- | --- | --- | --- | --- | --- | --- | --- |
| M2 | FANUC Mill 2 | 0.8% | 43.2% | 44% | 12.8h | 29h | ❌ Critical |
| M3 | FANUC Mill 3 | 0% | 0% | 0% | 0h | 29h | — Offline |
| M6 | FANUC Mill 6 | 24.2% | 30.7% | 54.9% | 15.9h | 29h | ❌ Critical |
| M8 | FANUC Mill 8 | 32.5% | 61.1% | 93.6% | 27.1h | 29h | ❌ Critical |
| T2 | YCM NTC1600LY | 2.5% | 76.3% | 78.8% | 22.9h | 29h | ❌ Critical |

# Daily Utilization Trend

## Daily Figures

| Date | M2 | M6 | M8 | T2 |
| --- | --- | --- | --- | --- |
| Mon, 2/2 | 0% / 0% | 0% / 0% | 39.2% / 86% | 2.2% / 64.2% |
| Tue, 2/3 | 1.8% / 75.1% | 46.5% / 99.5% | 33.2% / 100% | 3.5% / 99.4% |
| Wed, 2/4 | 0% / 100% | 32.2% / 100% | 0% / 98.9% | 0% / 52.8% |

# Running vs. Idle Hours

# Notes & Methodology

Data collected via FOCAS2 protocol, polling each machine every 60 seconds from collector PC (WrkStationC).

Utilization is defined as the percentage of shift-hour samples where the machine spindle speed > 0 or run status = STRT/MSTR.

Shift hours: 6:00 - 19:00, Monday through Friday.

M3 (FANUC Mill 3) is currently offline due to a failed Ethernet adapter. A USB-to-Ethernet replacement is on order.

Machines M4, M5, and M7 (Robodrills) are not yet connected to the monitoring network.

M1 (Haas Classic) uses a proprietary protocol and is not FOCAS-compatible.